Name:  _____________________________ (please print)

Signature: __________________________

ECE 2300 – Final Exam

May 9, 2015

Keep this exam closed until you are told to begin.

1.  This exam is closed book, closed notes.  You may use one 8.5” x 11” crib sheet, or its equivalent.

2.  Show all work on these pages.  Show all work necessary to complete the problem.  A solution without the appropriate work shown will receive no credit.  A solution that is not given in a reasonable order will lose credit.  Clearly indicate your answer (for example by enclosing it in a box).

3.  It is assumed that your work will begin on the same page as the problem statement.  If you choose to begin your work on another page, you must indicate this on the page with the problem statement, with a clear indication of where the work can be found.  If your work continues on to another page, indicate clearly where your work can be found.  Failure to indicate this clearly will result in a loss of credit.

4.  Show all units in solutions, intermediate results, and figures.  Units in the exam will be included between square brackets.

5.  Do not use red ink.  Do not use red pencil.

6.  You will have 170 minutes to work on this exam.

1.  ________________/35

2.  ________________/30

3.  ________________/35

4.  ________________/35

5.  ________________/35

6.  ________________/30

Total = 200

Room for extra work

- {35 Points} Use the circuit shown to solve this problem.

- A voltmeter has a full-scale voltage of 10[V], and a meter resistance of 10[kΩ]. Connect the meter to the following circuit to read vM. What does the meter read?

- In order to measure iM, remove the meter. Modify the meter so that it can read currents up to 100[mA]. Draw the modified meter.

- Connect the modified meter, to the circuit to read iM. What does the meter read?

- What is the error in measuring iM?

Room for extra work

2.  {30 Points} Use the node-voltage method to write a complete set of equations that could be used to solve this circuit. Do not simplify the circuit. Do not attempt to simplify or solve your equations. Define all variables clearly.

Room for extra work

3.  {35 Points} Use the circuit shown to solve this problem.

- Find the Thévenin equivalent as seen by the iS1 source.  Draw the equivalent, with the iS1 source connected.

- Find the power delivered by the iS1 source in the circuit shown.

# Room for extra work

4.  {35 Points} The switches in this circuit had been open for a long time before 
t = 0, at which time the switches both closed.

a) Find the time when, for t > 0, the voltage vX is equal to 10[V].  If vX does not equal 10[V] for any time t > 0, then show how you know this is true.

b) Find the energy stored in inductor L1 a long time after t = 0.

Room for extra work

5.  {35 Points} The switch in this circuit was open for a long time before t = 0, and then closed at t = 0.

a)  Find a numerical expression for iX(t) as a function of time, t, for time long after the switch closed.

b)  Find iX(0-).

c)  Find iX(0+).

Room for extra work

6. {30 Points} The dc motor in the circuit shown can be modeled with a 5.6[H] inductor in series with a 47[] resistor.  The voltage across the motor is monitored using a 1000[V] full-scale voltmeter that has a meter resistance of 5[M].  The motor had been running for a long time, and then was shut down by opening the switch at t = 0.

a)  Find the voltage across the voltmeter right after the switch was opened, assuming that the models here all remain valid.

b)  Assume that an additional resistor RX  is placed in parallel with the voltmeter.  Find the value for RX  needed to make sure that the magnitude of the voltage across the voltmeter does not exceed 1000[V] in this circuit.

c)  For the case in part b), assuming the value you found for RX, find the energy dissipated in RX  for t > 0.