Silvaco I (will be posted on the web).

To access Silvaco please change the path i.e. in your .cshrc put the following:   /opt/silvaco-2015/bin.

These simulations will show effects in silicon oxidation known as concentration enhanced oxidation, dopant segregation, and oxidation enhanced diffusion.

From examples please select:

- Section 39 Athena Oxidation and then 39.3 anoex03.in Doping Dependent Oxidation.  Load the example to Deckbuild.

    - Run the example

    - In Tonyplot, from Tools use Ruler and measure oxide thickness in low and high doped regions (for clarity you can delete mesh from Display menu)

    - From Tools select Cutline and make 2 vertical cuts – one in low doped and one in high doped regions.

    - Return to the example *.in file and change phos to boron in the implantation statement

    - Run the example again

    - Repeat all the steps as done for phosphorus implantation example.

    - Explain the reasons for oxide thickness differences in high and low doped silicon and  the role of dopant type.

- Section 48 Athena 1D and choose 48.15an1dex15.in Phosphorus Simulation. Load the example to Deckbuild.

  - Run the example

  - In Tonyplot from Edit select Split Overlay and identify each plot vs. simulation condition (please note that Silvaco does not support 1D so the appearance of the plots cannot be changed).

  - Analyze (qualitatively) the dependence of oxidation enhanced diffusion and the charges it caused in the profiles and concentrations levels in SiO2 and Si.